|  | AUSGABE (des Beilageblattes) 2026-07-29 In diesem Decoder ist folgendes ZIMO Sound-Projekt geladen: Regelspurdiesellokomotive U50 – Version 30.23 |
| --- | --- |

The GE U50 was an eight-axle, 5,000 hp (3.7 MW) diesel locomotive built by GE Transportation Systems. They were twin-engined locomotives, combining two 2,500 hp (1,850 kW) diesel engines.

The U50 rode on four two-axle trucks, grouped in pairs linked by span bolsters, giving a wheel arrangement of B+B-B+B. The trucks and bolsters were re-used from scrapped UP turbine locomotives built by GE during the 1950s. The U50 was built in response to the Union Pacific Railroad's requirement, issued in the early 1960s, for a 15,000 hp (11,100 kW) 3-unit locomotive intended to replace the turbines.

Der Einbau der ZIMO Sound Decoder MX64x (H0) und MX69x (Großbahn) mit diesem Projekt kann in alle passenden Modelle erfolgen.

ACHTUNG: Nach dem Einbau des Decoders ist eine Messfahrt notwendig  –>  CV  # 302 = 75

In diesem Projekt sind folgende Funktionszuordnungen getroffen:

F-Taste                     Einrichtung                                               am Funktionsausgang            Sound-Funktion

| F0 | Licht | FA0 (gelb/weis) |  |
| --- | --- | --- | --- |
| F1 |  | FA1 |  |
| F2 |  |  |  |
| F3 |  |  | Horn1 |
| F4 | Rangiergang /ABV |  |  |
| F5 |  |  | Horn2 |
| F6 |  |  |  |
| F7 |  |  | Glocke |
| F8 |  |  | SOUND ON/OFF |
| F9 |  |  | Kompressor |
| F10 |  |  | Luft1 |
| F11 |  |  | Luft2 |
| F12 |  |  | Lüfter1 |
| F13 |  |  | Lüfter2 |
| F14 |  |  | Zisch |
| F15 |  |  |  |
| F16 |  |  |  |
| F17 |  |  |  |
| F18 |  |  |  |
| F19 |  |  |  |

Für Mehrfachtraktion in CV154 Bit 1 setzen!